FREE VINCI 
MECHANICAL CODE LOCK WITH SECURED ENTRANCE AND FREE EXIT

SPECIFICATIONS

The mechanical code lock shall be 100% mechanical and shall not require any batteries or electricity to function.

The mechanical code lock shall have a solid stainless steel day bolt to guarantee a secure locking.

The mechanical code lock shall, without any adjustments, be possible to install on square, round or flat profiles.

The mechanical code lock shall allow a key operated retraction of the self-latching bolt.

An anodised aluminium handle pair shall be standard included.

It shall be possible to install different handle pairs on the mechanical code lock.

The mechanical code lock shall have one mechanical code panel to controlled entrance while the other side allows a free exit.

The mechanical code lock shall have a polyamide dust and rain cover.

The mechanical code lock shall have an adjustable rollerbolt for easy closing.

The mechanical code lock shall allow an automatic code reset after pushing the handle.

The mechanical code lock shall have an anti-code detection thanks to an integrated security press plate.

The mechanical code lock shall be available in Silver or RAL 6005, 6009, 7016, 9005 and 9010.

The mechanical code lock should be designed and manufactured in Europe or in the United States.

The mechanical code lock shall allow to close the free exit side of the gate.

PERFORMANCE

The mechanical code lock shall be specifically made for outdoor use.

The mechanical code lock shall be maintenance free (no greasing).

There shall be no recognizable wear and tear to the push buttons after high usage number combinations.

The mechanical code lock shall have been tested for 500.000 movements.

The manufacturer warranty shall be 3 years.

INSTALLATION

No welding shall be required to install the mechanical code lock.

The mechanical code lock shall be installed with a 4-hole pattern with 2 hexagon socket head screws.

The mechanical code lock shall allow an easy and quick configuration or modification of the codes.

The self-latching daybolt of the mechanical code lock shall be able to be used for both left and right turning gates.

The mechanical code lock shall allow a 20mm (3/4”) continuous adjustability of the bolt for a perfect alignment of the locking mechanism.

An installation explanatory video shall be available to assist the installer.

An optional drilling jig (Drill-Fix) should be recommended for a correct installation.

MATERIAL

The housing of the mechanical code lock shall be KTL / E-coated.

The mechanical code lock  shall have a corrosion resistance of 500h salt spray according to ISO9227.

The mechanical code lock shall be made of a vandal-proof, powder coated aluminium housing (no wet painting or anodization). The powder coating shall be according to the Qualicoat standards.

The mechanical code lock shall have a stainless steel mechanism.

The mechanical code lock shall have stainless steel push buttons.

The mechanical code lock shall be fully weather- and dustproof .

The mechanical code lock shall have an UV-resistance against discoloration of 500h.