ELECTRADROP-E & ELECTRADROP-E-P
MOTORISED ELECTRICAL DROPBOLT

SPECIFICATIONS

Operated by a motorised leadscrew.

Available in two colours : silver or black anodization.

Obstacle detection system (200N) in downwards movement with automatic stop and reverse.

Allows a mechanical release (by key) in case of emergency or power failure.

Can be connected to a warning light on automated gates (min. 3sec pre-flash).

Fitted with a relay if the control signal is out of range (>24V).

Throw of 120mm (4 – 12/16”).

Diameter of the dropbolt is 20mm (13/16”).

Able to be fitted on gate profiles of minimum 40mm wide (1 – 9/16”).

Dropbolt lock designed and manufactured in Europe or in the United States.

PERFORMANCE

Resistance for horizontal pressure on the latch bolt of 1000kg without functional damage.

Tested for 500.000 movements.

2 years warranty.

INSTALLATION

Quick-fix bolts pre-installed in the product.

Can be installed vertically (bolt pointing downwards) and horizontally.

5m connection cable included.

3D-installation video available to assist the installer.

ELECTRICAL

Electrical requirements

  - Power voltage: 24V DC

  - Consumption: Normal: 25 W / Peak: 55W / Standby: 0,2 W

  - Control signal (output warning lamp): 12-24V AC/DC

MATERIAL

Vandal-proof, anodized aluminium housing.

Corrosion resistance of 500h salt spray according to ISO9227.

Fully weather- and dustproof (IP66).

UV-resistance against discoloration of 500h.

ELECTRADROP-R & ELECTRADROP-R-P
MOTORISED ELECTRICAL DROPBOLT

SPECIFICATIONS

Operated by a motorised leadscrew.

Available in two colours : silver or black anodization.

Obstacle detection system (200N) in downwards movement with automatic stop and reverse.

Allows a mechanical release (by key) in case of emergency or power failure.

Can be connected to a warning light on automated gates (min. 3sec pre-flash).

Fitted with a relay if the control signal is out of range (>24V).

Throw of 120mm (4 – 12/16”).

Diameter of the dropbolt is 20mm (13/16”).

Able to be fitted on gate profiles of minimum 40mm wide (1 – 9/16”).

Automatic lift of the dropbolt by integrated battery pack in case of power failure.

>300 bolt movements on 2x 9V LiMnO2  batteries with 10 years battery lifetime.

Integrated LED for signal feedback and low battery warning.

Dropbolt lock designed and manufactured in Europe or in the United States.

PERFORMANCE

Resistance for horizontal pressure on the latch bolt of 1000kg without functional damage.

Tested for 500.000 movements.

2 years warranty.

INSTALLATION

Quick-fix bolts pre-installed in the product.

Can be installed vertically (bolt pointing downwards) and horizontally.

5m connection cable included.

3D-installation video available to assist the installer.

ELECTRICAL

Electrical requirements

  - Power voltage: 24V DC

  - Consumption: Normal: 25 W / Peak: 55W / Standby: 0,2 W

  - Control signal (output warning lamp): 12-24V AC/DC

MATERIAL

Vandal-proof, anodized aluminium housing.

Corrosion resistance of 500h salt spray according to ISO9227.

Fully weather- and dustproof (IP66).

UV-resistance against discoloration of 500h.