VITA
MECHANICAL SURFACE-MOUNTED CODE LOCK FOR INSERT LOCKS

Specification text: The gate will be fitted with an insert lock and a Locinox VITA mechanical code keypad (or a technically equivalent product) designed specifically for durable outdoor use. The mechanical design requires no wiring or battery and is completely maintenance-free. The device is corrosion-resistant (withstands over 1,000 hours in a salt spray test according to the ISO 9227 standard) and suitable for intensive use thanks to robust mechanical components designed for more than 500,000 operating cycles. Installation requires minimal preparation of the gate profile and allows the access code to be changed without dismantling the lock or keypad.

FEATURES

Mechanical surface-mounted code lock

Can be combined with insert lock

Suitable for mounting on a flat surface (either on the gate profile or on the gate cladding).

Different handle options available

Choice of either single-sided code keypad with cover plate or double-sided code keypad

Different input and output codes possible

Free exit via handle

Automatic code reset after handle operation

Anti-code detection feature provided by the integrated security plate

Available in both silver and RAL 9005 black

Designed and manufactured in Europe for the USA

SPECIFICATIONS

Designed for outdoor use

Tested to withstand over 500,000 operations

Maintenance free

Three-year factory warranty

INSTALLATION

Mounting via two bolts

Simple code setup

Installation video available

MATERIAL

Aluminum housing.

  - Black version: fine textured powder coating based on the Qualicoat standard

  - Silver version: anodization

Rust resistance: 1,000-hour salt spray test according to ISO 9227

Push buttons:

  - Black version: black anodized aluminum

  - Silver version: stainless steel