INTERIO
INVISIBLE, BUILT-IN HYDRAULIC GATE CLOSER

SPECIFICATIONS

The gate closer shall be built-in invisibly in the gate profile ensuring a vandal-proof use.

The gate closer shall have matching top and bottom hinge sets.

Optional hinge sets for ground and wall installation shall be available.

The gate closer shall be 100% mechanical (no electronic components).

It shall be possible to adjust the closing speed of the gate by simply adjusting a valve with an Allen key, and this in a continuous way to ensure micro-adjustments.

It shall be possible to activate a final snap (free run) on the gate closer to ensure the latching of the gate.

It shall be possible to eliminate the final snap (free run) on the gate closer completely in order to have a silent closing (i.e. in combination with magnetic locks).

The closing speed shall be adjustable.

The gate closer shall allow for a 180 degrees opening angle of the gate.

When opened 180 degrees, the gate closer should self-close over the 180 degrees.

The gate closer shall have a hydraulic dampening mechanism.

The gate closer shall be designed and manufactured in Europe or in the United States.

The gate closer shall be ADA, ROSPA and PMR compliant.

The gate closer shall have double rubber sealings to avoid oil leakage at all times.

The gate closer shall use rotary seals (vs. linear) seals to ensure the longevity of the closer.

The gate closer shall be able to close gates up to 150 kilo / 330 lbs and gate width up to 1,5m / 3.937 ft.

PERFORMANCE

The gate closer shall be specifically made for outdoor use.

The gate closer shall be easy to open with a max force of 20Nm.

The hydraulic dampening shall be guaranteed to work properly under all temperature circumstances without any summer or winter adjustments, measured at a 90° opening. The viscosity of the oil shall have no impact on the performance of the gate closer.

  - The closing time shall not be below 10 seconds at 70 degrees Celsius / 185 degrees Fahrenheit (summer).

  - The closing time shall not be longer than 30 seconds at -30 degrees Celsius / -22 degrees Fahrenheit (winter).

The closer shall have been tested for 500,000 movements.

The closer shall be maintenance free (no greasing nor oil refill).

The manufacturer warranty shall be 3 years.

INSTALLATION

A pre-tension shall be integrated in the gate closer ensuring an efficient installation.

The closer shall have pre-mounted fasteners (no welding) with stainless steel bolts. All brackets for fixation should come with the product.

The same gate closer shall be able to be used for both left and right turning gates.

Depending of the installation, the gate closer shall require a gate stop to be used for limiting the opening angle.

A 3D-installation explanatory video shall be available for assisting the installer.

Drilling templates shall be included for assisting the installer.

MATERIAL

The mechanism shall be made out stainless steel and aluminum components.

The housing of the mechanism shall be made from extruded, powder coated aluminum (no wet painting or anodization). The powder coating shall be according to the Qualicoat standards.

The gate closer shall have a corrosion resistance of 500h salt spray according to ISO9227.

The gate closer shall be fully weather- and dustproof.

The gate closer shall have a UV-resistance against discoloration of 500h.